SOFTENG 364 Assignment 1

840454023, elee353

Task 1

Task 2

Q1.

The highest packets-per-second value seen was 87 packets per second.

It occurred in the 20th second.

Q2.

The highest bits-per-second value is 623500 bits per second.

It occurred in the 20th second.

Q3.

tcp.analysis.duplicate_ack

tcp.analysis.fast_retransmission

tcp.analysis.retransmission

Q4.

Duplicate ACKs are sent when the receiver sees a gap in the packets it receives. The gap can be caused by a lost segment or just a reordering of segments.

The sender will wait for a small number of duplicate ACKs to be received. If there is only one or two duplicate ACKs received before the reordered segment is processed, there is just a reordering of the segments and a new ACK will be generated.

If three or more duplicate ACKs are received in a row, it is a strong indication that a segment has been lost.

Q5.

There are 2584 TCP conversations.

Q6.

"Tcp previous segment not captured" means Wireshark did not see a packet that should have been in the trace.

This is caused by either the packet really wasn't seen on the wire due to a packet loss, or Wireshark did not record the packet fast and timely even though it had been on the wire.

There is 172 times “previous segment not captured” has been detected.

Q7.

There are 183 retransmissions.

There are 59 fast transmissions.

Task 3

Q1.

- ARP, NBNS, BROWSER, CLDAP, DB-LSP-DISC, DHCP, DNS, HTTP, OCSP, QUIC, SSDP, TCP, TLSv1, TLSv1.2, UDP

- 0 percent of the total was involved with DCHP messages.

- Both TCP and UDP are used here.

Q2.

- All packets

- Bootstrap

- DNS

- TCP

- UDP

Q3.

The highest bits-per-second rate is 113700 bits per second.

It occurred in the 146th second.

Q4.

Request IP: 192.168.1.73

Response IP: 69.164.192.146

Q5.

- TCP conversation

Q6.

Download time = 192.372 – 169.269 = 23.103 seconds

Q7.

Q8.

The longest RTT occurred in packet 17280, in the 182.268th second.

Task 4

- General

Video traffic is the major consumer of internet bandwidth, because multimedia files are generally large. Videos are a sequence of images displayed at a constant rate, while digital images are arrays of pixels.

- Netflix

- YouTube

- Lightbox

- CDN, DASH

- Discuss how video streaming on the internet is achieved in general

- then discuss the approaches taken by the above three services

- the protocols involved

- facts pertaining to New Zealand

- Pictures

- reference all your sources, including any images taken from other sources

- 1200 words